Technical Specifications

XF

|  | XF 2.2D | XF 3.0L V6D | XF 3.0L V6D S | XF 3.0L V6 | XF 5.0L V8 NA | XF 5.0L V8 R |
| --- | --- | --- | --- | --- | --- | --- |
| ENGINE & TRANSMISSION |  |  |  |  |  |  |
| Engine Capacity (cc) | 2179 | 2993 | 2993 | 2967 | 5000 | 5000 |
| Cylinders | 4 | 6 | 6 | 6 | 8 | 8 |
| Valves per Cylinder | 4 | 4 | 4 | 4 | 4 | 4 |
| Compression Ratio | 15.8:1 | 16.0:1 | 16.0:1 | 10.5:1 | 11.5:1 | 9.5:1 |
| Transmission | 8-speed automatic | 8-speed automatic | 8-speed automatic | 6-speed automatic | 6-speed automatic | 6-speed automatic |
| PERFORMANCE |  |  |  |  |  |  |
| 0-60mph (seconds) | 8.0 | 6.7 | 5.9 | 7.9 | 5.5 | 4.7 |
| 0-100km/h (seconds) | 8.5 | 7.1 | 6.4 | 8.3 | 5.7 | 4.9 |
| Top Speed (mph / km/h) | 140/225 | 149/240 | 155/250 | 148/237 | 155/250 | 155/250 |
| Max. Power (SAE bhp) @ rpm | 187@3500 | 237@4000 | 271@4000 | 235@6500 | 380@6500 | 503@6000-6500 |
| Max. Power (EEC PS) @ rpm | 190@3500 | 240@4000 | 275@4000 | 238@6500 | 385@6500 | 510@6000-6500 |
| Max. Power (EEC kW) @ rpm | 140@3500 | 177@4000 | 202@4000 | 175@6500 | 283@6500 | 375@6000-6500 |
| Torque (SAE lb ft) @ rpm | 332@2000 | 369@2000 | 442@2000 | 216@4100 | 380@3500 | 461@2500-5500 |
| Torque (EEC Nm) @ rpm | 450@2000 | 500@2000 | 600@2000 | 293@4100 | 515@3500 | 625@2500-5500 |
| Aerodynamics (Cd) | 0.29 | 0.29 | 0.29 | 0.29 | 0.29 | 0.29 |
| ECONOMY |  |  |  |  |  |  |
| Combined Fuel Economy (mpg) | 52.3 | 44.8 | 44.8 | 26.8 | 25.4 | 22.5 |
| Combined Fuel Economy (l/100km) | 5.4 | 6.3 | 6.3 | 10.5 | 11.1 | 12.5 |
| US C-H Combined (mpg) | n/a | n/a | n/a | n/a | 19 | 17 |
| NEDC Combined C02 (g/km) | 149 | 169 | 169 | 249 | 264 | 292 |
| Emissions Standard | EU5 | EU5 | EU5 | EU4 | ULEVII/TIERII BIN5/EU5 |  |